| S.No. | Reference | Component / Item | Function | Manufacturer | MPN / Model | Specification / Value | Qty | Footprint / Module | Verification Status | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | U1 | ESP32 Development Board | Main microcontroller | Espressif | ESP32-DEVKITC-32E | ESP32 development board | 1 | Development board | Confirmed | Board family identified from the project schematic. Verify exact physical board revision if required. |
| 2 | U2 | BME280 | Temperature, humidity and pressure sensing | Bosch Sensortec | BME280 | Environmental sensor; I²C/SPI | 1 | Breakout/module | Sensor confirmed; module TBD | Exact breakout-board manufacturer/MPN should be taken from the physical module. |
| 3 | U3 | SGP30 | TVOC and eCO₂ sensing | Sensirion | SGP30 | Digital gas sensor; I²C; TVOC/eCO₂ | 1 | Breakout/module | Confirmed final sensor | SGP30 is the actual sensor used in HAQI-SMART. The KiCad U3 symbol should be reviewed because the supplied schematic appears inconsistent with the actual SGP30. |
| 4 | U5 | MQ-7 | COx / CO-related gas sensing | TBD | MQ-7 | Gas sensor; analog output | 1 | Sensor/module | Sensor confirmed; module TBD | Verify exact MQ-7 module manufacturer and part number from the physical unit. |
| 5 | — | PMS7003 | Particulate-matter sensing | Plantower | PMS7003 | Particulate-matter sensor; UART | 1 | Sensor/module | Confirmed | Final project uses PMS7003. |
| 6 | U6 | OLED Display | Local live-data display and visualization | TBD | TBD | I²C OLED; live PM2.5, temperature, humidity, pressure, TVOC, eCO₂ and MQ-7-related value | 1 | OLED module | Function confirmed; exact model TBD | The supplied schematic appears to identify an ER-OLEDM0.91-1-I2C family. Verify the actual physical display before entering final MPN. |
| 7 | U7 | microSD Module + microSD Card | Local data logging | TBD | TBD | SPI microSD interface | 1 | microSD module | Function confirmed; exact module/card TBD | Record actual module model and card capacity. |
| 8 | — | Solar Panel | Solar energy generation | TBD | TBD | 10 W | 1 | Solar panel | Power rating confirmed; exact model TBD | Final prototype uses a 10 W panel. Verify rated voltage/current from the panel label/datasheet. |
| 9 | — | Solar Charge Controller | Solar battery charging | TBD | TBD | Solar charging controller | 1 | Controller module | Function confirmed; exact model TBD | Exact controller and battery compatibility must be verified. |
| 10 | — | 18650 Battery Pack | Rechargeable energy storage | TBD | TBD | Li-ion battery pack; exact configuration/capacity TBD | 1 | Battery pack | Confirmed; exact specification TBD | Earlier development records indicate a 7.4 V-class pack; verify final physical pack before recording voltage/capacity. |
| 11 | — | BMS | Battery protection / management | TBD | TBD | Li-ion BMS; exact configuration/rating TBD | 1 | BMS module | Confirmed; exact specification TBD | Exact BMS must match the final battery configuration. |
| 12 | — | Buck Converter | Voltage regulation | TBD | TBD | DC-DC step-down; regulated system supply | 1 | Converter module | Confirmed; exact model/rating TBD | Record actual input range, configured output voltage and current rating. |
| 13 | — | YX850 Power-Path / Switching Module | Power switching / path management | TBD | YX850 | Automatic power switching / power-path module | 1 | Module | Model confirmed; exact vendor/revision TBD | Final battery-side path: 18650 battery → buck converter → YX850 → ESP32/system electronics. |
| 14 | R1 | Resistor | Green LED current limiting | TBD | TBD | 330 Ω | 1 | Resistor | Value confirmed from schematic | Schematic shows R1 = 330R. |
| 15 | R2 | Resistor | Red LED current limiting | TBD | TBD | 330 Ω | 1 | Resistor | Value confirmed from schematic | Schematic shows R2 = 330R. |
| 16 | D1 | Green LED | Status indication | TBD | TBD | Green LED | 1 | LED | Type/color confirmed from schematic | Exact package/manufacturer not specified. |
| 17 | D2 | Red LED | Status indication | TBD | TBD | Red LED | 1 | LED | Type/color confirmed from schematic | Exact package/manufacturer not specified. |