# HAQI-SMART

## Hardware Design History

Project: HAQI-SMART — Smart Air Quality Monitoring System
 Documentation Domain: Hardware / Electrical
 Document Type: Hardware Design History
 Revision: V1.0
 Status: Draft — Hardware Team Review

# 1. Purpose of This Document

This document records the evolution of the HAQI-SMART hardware from the initial concept through the final prototype architecture.

The purpose of maintaining a design history is to document:

- how the hardware architecture evolved,

- which components were considered or tested,

- why components were changed or retained,

- how the power architecture developed,

- major integration decisions,

- problems encountered during development,

- and the reasoning behind the final hardware configuration.

The history is intended to provide context for the final hardware design rather than presenting only the final list of components.

# 2. Initial Hardware Concept

The initial hardware concept for HAQI-SMART was based on developing a low-cost, portable air-quality monitoring system capable of collecting multiple environmental and air-quality parameters.

The initial architecture required four major capabilities:

- Air-quality sensing

- Environmental sensing

- Local visualization

- Data storage

A microcontroller was required to coordinate the sensors, process their readings and provide the user interface.

The hardware development therefore began by evaluating suitable sensors and a microcontroller capable of integrating multiple peripherals.

# 3. Microcontroller Selection

## 3.1 Initial Controller Consideration

Different microcontroller platforms were considered during the early hardware-development stage, including ESP8266 and ESP32-based approaches.

The ESP32 was eventually selected as the primary controller.

## 3.2 Reason for Selection

The ESP32 was preferred because the project required several hardware interfaces simultaneously.

The final system required connections for:

- multiple I²C devices,

- a UART particulate sensor,

- an analog gas sensor,

- an SPI-based microSD interface,

- and an OLED display.

The ESP32 also provides integrated wireless capability, making it suitable for future expansion toward connected/cloud-based monitoring.

## 3.3 Final Decision

The final hardware architecture uses an ESP32-DEVKITC-32E development board as the central controller. This is also identified as U1 in the supplied KiCad schematic.

# 4. Development of the Environmental-Sensing Subsystem

## 4.1 Initial Humidity/Temperature-Sensing Consideration

During early sensor evaluation, a DHT22/AM2302 sensor was considered/used for environmental measurement.

However, the sensor's response behaviour was found to be less suitable for the desired system, particularly because of its relatively slow response.

This led to consideration of a more capable environmental sensor.

## 4.2 Transition to BME280

The BME280 was selected for the final design.

The BME280 provided:

- temperature,

- humidity,

- and atmospheric pressure.

This represented an improvement in the environmental-sensing subsystem because it added pressure measurement while providing a digital interface suitable for integration with the ESP32.

### Final decision

DHT22/AM2302 → BME280

The BME280 became part of the final hardware configuration.

# 5. Development of Gas-Sensing Subsystem

## 5.1 Initial Gas-Sensor Considerations

Gas sensing was considered an important part of HAQI-SMART because particulate-matter measurements alone would not provide information about gaseous pollutants.

During the development process, different MQ-series gas-sensing options were considered.

MQ-135 was among the sensors discussed during the earlier development stage.

However, it was not retained as part of the final hardware configuration.

## 5.2 Transition to MQ-7

The project ultimately used the MQ-7.

The MQ-7 was selected to provide the project's COx/CO-related gas-sensing channel.

This provided a more specific gas-sensing purpose than the broader MQ-135 approach that had previously been considered.

### Final decision

MQ-135 considered → MQ-7 used in final prototype

The MQ-7 was integrated as an analog sensor connected to the ESP32.

# 6. Addition of SGP30

The sensing architecture was expanded further with the inclusion of the SGP30.

The SGP30 was selected to provide:

- TVOC

- eCO₂

This complemented the MQ-7 gas-sensing channel.

The resulting gas/air-quality architecture therefore became capable of providing multiple categories of information:

| Sensor | Measurement |
| --- | --- |
| MQ-7 | COx/CO-related gas sensing |
| SGP30 | TVOC |
| SGP30 | eCO₂ |
| PMS7003 | Particulate matter / PM2.5 |

This multi-sensor approach became an important part of the final HAQI-SMART architecture.

# 7. Particulate-Matter Sensor Development

The project required a dedicated particulate-matter sensor because particulate pollution could not be measured using the gas sensors.

The PMS7003 was selected for particulate-matter monitoring.

The PMS7003 became the dedicated particulate-sensing component, with PM2.5 being one of the principal parameters used by the system.

The PMS7003 communicates with the ESP32 through a serial/UART interface.

# 8. Development of the Display Subsystem

An OLED display was incorporated into the system to provide local visualization.

The purpose was to ensure that the prototype could present sensor information directly without requiring a separate computer.

The display functionality evolved beyond simply showing a single sensor value.

The final OLED interface was designed to provide live information including:

- PM2.5

- temperature

- humidity

- pressure

- TVOC

- eCO₂

- MQ-7-related value

The display can additionally support:

- air-quality indicators,

- animations,

- live-data presentation,

- and other visual feedback.

This made the OLED an important part of the final user-interface architecture.

# 9. Development of Local Data Logging

Real-time display alone was insufficient for a monitoring system intended to collect measurements over time.

A microSD storage subsystem was therefore incorporated.

The microSD card provides persistent local storage for the sensor readings.

The ESP32 manages the data-logging process, allowing measurements to be retained for subsequent:

- analysis,

- comparison,

- visualization,

- and documentation.

The microSD subsystem therefore became part of the final hardware architecture.

# 10. Initial Power-System Development

Power was one of the more involved parts of the hardware-development process.

The system needed to support:

- external DC power,

- battery operation,

- portable operation,

- and eventually solar charging.

The initial prototype/testing stages relied significantly on external DC power sources such as an adapter and other regulated supplies.

As the project progressed, a rechargeable battery system was introduced to provide portable operation.

# 11. Introduction of 18650 Battery Power

The project adopted an 18650 lithium-ion battery system for energy storage.

The battery allowed the prototype to operate without remaining permanently connected to a DC adapter.

This introduced several additional design requirements:

- battery protection,

- voltage regulation,

- charging,

- and controlled power delivery.

A BMS was therefore incorporated as part of the battery subsystem.

# 12. Development of the Solar-Power Subsystem

The project subsequently incorporated solar energy harvesting to improve the portability and renewable-energy capability of the prototype.

Different solar-panel specifications were considered during the development process.

The final prototype uses a 10 W solar panel.

The solar panel was integrated with a solar charging subsystem rather than being connected directly to the battery.

The resulting architecture introduced:

Solar Panel → Solar Charge Controller → Battery System

as the charging path.

# 13. Development of the Voltage-Regulation Stage

The battery voltage was higher than the regulated voltage required by the low-voltage electronics.

A buck converter was therefore introduced to step down the battery voltage.

During development, the battery voltage was observed at approximately 7.6 V under one of the relevant testing conditions, while the intended regulated output was approximately 5 V.

The buck converter was consequently positioned between the battery and the downstream electronics/power-management stage.

# 14. Buck-Converter Adjustment Issue

During the development of the power subsystem, the buck converter presented an adjustment problem.

The output was initially around 7 V or higher, and repeated adjustment of the converter potentiometer did not bring the output down to the intended approximately 5 V level.

The adjustment mechanism was turned substantially during troubleshooting.

This became one of the documented hardware issues and highlighted the importance of:

- verifying the converter model,

- confirming its adjustment range,

- checking its electrical specifications,

- and measuring its output before connecting the complete system.

The issue is documented separately in the Hardware Known Issues & Errata document.

# 15. Development of Power-Path / Switching Architecture

As the project moved toward a portable system capable of operating from different power sources, a controlled switching/power-path solution became necessary.

The YX850 power-path/switching module was incorporated into the final architecture.

The module provides the switching/power-path stage between the regulated power source and the ESP32-side electronics.

The final battery-side arrangement became:

18650 Battery → Buck Converter → YX850 → ESP32/System Electronics

This arrangement was adopted instead of attempting to connect different power sources directly together.

# 16. OLED and Power-Integration Problems

During power-system integration, abnormal OLED behaviour was observed.

When the system was operated from battery power, the OLED did not display the expected interface and instead exhibited a green/distorted screen.

When the system was initially powered from a DC adapter, the OLED operated normally.

This observation led to further investigation of the power subsystem.

During subsequent testing, the OLED later stopped operating even when the system was supplied from the DC adapter.

The exact root cause was not conclusively established from the available development measurements.

These observations became an important part of the power-integration troubleshooting process.

# 17. Evolution Toward the Final Hardware Architecture

After sensor selection, interface development and power-system iterations, the hardware architecture converged toward the following structure.

## Sensing

BME280

→ Temperature / Humidity / Pressure

SGP30

→ TVOC / eCO₂

MQ-7

→ COx/CO-related gas sensing

PMS7003

→ Particulate matter / PM2.5

↓

## Processing

ESP32

↓

## Output and Storage

OLED

→ Live data + air-quality visualization

microSD

→ Local data logging

## Power

10 W Solar Panel

↓

Solar Charge Controller

↓

18650 Battery + BMS

↓

Buck Converter

↓

YX850 Power-Path / Switching Module

↓

ESP32/System Electronics

# 18. Final Hardware Configuration

The final hardware configuration established during development consists of:

| Component | Final role |
| --- | --- |
| ESP32-DEVKITC-32E | Main controller |
| BME280 | Temperature, humidity and pressure |
| SGP30 | TVOC and eCO₂ |
| MQ-7 | COx/CO-related gas sensing |
| PMS7003 | Particulate matter / PM2.5 |
| OLED | Live local display and visualization |
| microSD | Local data logging |
| 10 W solar panel | Solar energy generation |
| Solar charge controller | Battery charging |
| 18650 battery | Energy storage |
| BMS | Battery protection/management |
| Buck converter | Voltage regulation |
| YX850 | Power switching/power-path |

The supplied KiCad schematic represents the electrical integration of the principal sensing, processing, display and storage components.

# 19. Major Design Changes Summary

| Development stage | Earlier approach / consideration | Final approach | Reason for change |
| --- | --- | --- | --- |
| Microcontroller | ESP8266 considered | ESP32 | More suitable interface and processing capability |
| Environmental sensor | DHT22/AM2302 considered/used during early development | BME280 | More capable environmental measurement and additional pressure sensing |
| Gas sensor | MQ-135 considered | MQ-7 | Final requirement for COx/CO-related sensing |
| VOC sensing | Not initially sufficient as a standalone gas channel | SGP30 added | TVOC/eCO₂ measurement |
| Particulate sensing | Dedicated PM sensing required | PMS7003 | PM2.5/particulate monitoring |
| Display | Local display required | OLED | Live data and visual feedback |
| Data storage | Real-time readings alone insufficient | microSD | Persistent local logging |
| Power source | External DC supply/testing | Battery + solar capability | Portable and renewable operation |
| Battery regulation | Direct battery voltage unsuitable | Buck converter | Regulated lower-voltage supply |
| Power management | Multiple sources required controlled handling | YX850 power-path/switching | Controlled power-source management |
| Solar panel | Earlier specifications considered | 10 W final panel | Final prototype configuration |

# 20. Key Engineering Lessons From the Development

The hardware development produced several important engineering lessons.

### 20.1 Component Selection Must Consider the Complete System

A sensor cannot be selected only on the basis of its measurement capability.

Its:

- communication interface,

- voltage requirements,

- response characteristics,

- software support,

- physical integration,

- and interaction with other components

must also be considered.

### 20.2 Power Architecture Requires Early Planning

The development demonstrated that a portable system requires more than simply adding a battery.

The battery subsystem must consider:

- charging,

- protection,

- voltage regulation,

- power switching,

- source transitions,

- and load behaviour.

### 20.3 Different Power Sources Can Produce Different System Behaviour

The OLED behaviour during battery-powered versus DC-adapter operation demonstrated that the complete system must be validated under every intended power condition.

### 20.4 Low-Cost Sensors Require Careful Interpretation

The MQ-7 and other low-cost sensing elements should not automatically be treated as calibrated laboratory instruments.

Calibration, environmental conditions, sensor behaviour and data processing must be considered before making quantitative claims.

### 20.5 Modular Architecture Simplifies Development

Using separate modules for:

- sensing,

- processing,

- display,

- storage,

- power conversion,

- and power switching

allowed individual sections of the prototype to be tested and modified without redesigning the entire system.

# 21. Final Design-History Statement

The HAQI-SMART hardware evolved through iterative sensor evaluation, interface integration, power-system development and troubleshooting.

The final architecture was not established in a single step. It developed progressively as the requirements for multi-parameter sensing, local visualization, data logging, portability and solar-powered operation became clearer.

The major evolution was from a basic sensor-and-microcontroller concept toward a complete integrated system consisting of:

Multi-parameter sensing → ESP32 processing → OLED visualization + microSD logging

supported by:

Solar energy → battery storage/protection → voltage regulation → power-path switching → system electronics

The final hardware therefore represents the result of iterative engineering decisions and practical prototype testing.

# 22. Documentation Status

Document: HAQI-SMART Hardware Design History
 Revision: V1.0
 Status: Draft — Hardware Team Review

### Primary references

- Final KiCad electrical schematic supplied by the hardware team.

- Hardware development records and confirmed project information.

### Items requiring confirmation before final submission

The exact dates of individual design changes, exact dates of component replacement/testing, and exact reasons for some early component-selection decisions were not formally recorded in the available project material.

Therefore, this document intentionally records the technical progression without inventing dates or undocumented reasons.