# HAQI-SMART

## Hardware Known Issues & Errata

Project: HAQI-SMART — Smart Air Quality Monitoring System
 Documentation Domain: Hardware / Electrical
 Document Type: Hardware Known Issues & Errata
 Revision: V1.0
 Status: Hardware Development Record

# 1. Purpose of This Document

This document records hardware-related problems and abnormal behaviour observed during the development, integration and testing of the HAQI-SMART prototype.

The purpose of maintaining this record is to:

- preserve the development history of the hardware,

- document abnormal behaviour that occurred during testing,

- record the symptoms observed,

- document the investigation performed,

- distinguish confirmed causes from suspected causes,

- record corrective actions or workarounds,

- and provide useful information for future development and troubleshooting.

The issues documented here are based on the actual problems encountered during the HAQI-SMART hardware development process.

Where the exact root cause was not conclusively established, the issue is explicitly marked as cause not conclusively determined rather than assigning an unsupported explanation.

# 2. Issue Classification

The hardware issues are classified as follows:

| ID | Issue | Category | Status |
| --- | --- | --- | --- |
| HW-01 | OLED abnormal display behaviour during battery operation | Display / Power | Investigated |
| HW-02 | OLED stopped operating during later DC-adapter testing | Display / Power | Investigated |
| HW-03 | Buck converter output could not initially be adjusted to required voltage | Power Regulation | Investigated |
| HW-04 | Power architecture required multiple iterations during development | Power Architecture | Resolved through design iteration |

# 3. HW-01 — OLED Abnormal Behaviour During Battery Operation

## 3.1 Description

During testing of the HAQI-SMART prototype using battery power, the OLED display did not operate normally.

Instead of displaying the expected text and interface information, the display appeared to show a green screen with visible distortion.

This behaviour was different from the behaviour observed when the system was powered using a DC adapter.

## 3.2 Conditions Under Which the Issue Was Observed

The issue was observed under the following conditions:

Power source: Battery

Expected behaviour: OLED displays text/data normally.

Observed behaviour: OLED screen became green with visible distortion and did not display the expected interface correctly.

## 3.3 Comparison With DC Power

An important observation during troubleshooting was that the OLED behaved normally when the system was powered from a DC adapter.

Therefore, the problem appeared to be associated with the battery-powered operating condition rather than being an immediately reproducible OLED failure under all power conditions.

This comparison was useful because it narrowed the investigation toward the power-delivery path or operating conditions associated with battery operation.

## 3.4 Investigation

The behaviour was compared between two supply conditions:

| Condition | OLED behaviour |
| --- | --- |
| Battery-powered operation | Green/distorted display |
| DC-adapter operation | OLED initially operated normally |

This indicated that the display itself could operate under at least one supply condition.

However, the available development observations did not conclusively establish whether the underlying cause was:

- supply-voltage instability,

- voltage drop under load,

- switching/power-path behaviour,

- electrical noise,

- grounding,

- or another power-integrity issue.

Therefore, no single root cause should be claimed without further measurement.

## 3.5 Root Cause

Not conclusively determined during the documented testing.

The strong correlation with battery-powered operation indicated that the power-delivery path was an important area of investigation, but the available testing did not establish a definitive root cause.

## 3.6 Corrective / Investigative Action

The power architecture was subsequently examined and developed through multiple iterations involving:

- battery supply,

- voltage regulation,

- buck conversion,

- power switching,

- and power-source management.

The issue should therefore be considered part of the hardware power-integration development history.

## 3.7 Documentation Note

This issue should not be documented as proof that the OLED was defective.

The correct engineering conclusion from the observed behaviour is:

The OLED exhibited abnormal operation specifically during battery-powered testing, while it initially operated normally under DC-adapter power. The exact electrical root cause was not conclusively established.

# 4. HW-02 — OLED Later Failed to Operate During DC-Adapter Testing

## 4.1 Description

After the earlier battery-versus-DC behaviour was observed, the OLED subsequently stopped turning on even when the system was powered from the DC adapter.

This was a change from the earlier observation in which the OLED operated correctly under DC-adapter power.

## 4.2 Expected Behaviour

When the system was supplied by the DC adapter:

- the OLED should power on,

- initialize correctly,

- and display the programmed interface/data.

## 4.3 Observed Behaviour

During later testing:

Power source: DC adapter

Observed behaviour: OLED did not turn on.

This indicated that the earlier battery-specific symptom had evolved into a condition where the display was no longer operating even under the previously functional DC-powered condition.

## 4.4 Investigation

The power output was subsequently checked during troubleshooting.

The development process therefore moved from simply observing the display behaviour toward examining the electrical supply and power-conversion stages.

However, the available project record does not provide enough evidence to state a definitive failure mechanism for the OLED.

## 4.5 Root Cause

Not conclusively determined.

Potential areas requiring investigation included:

- supply voltage,

- power-converter behaviour,

- power-path switching,

- wiring,

- grounding,

- OLED module condition,

- and electrical stress.

These should remain listed as investigation areas rather than being presented as confirmed causes.

## 4.6 Recommended Engineering Record

For future reproduction/testing, the following should be recorded whenever this issue is investigated again:

- voltage measured directly at the OLED VCC and GND,

- voltage at the ESP32 supply,

- voltage before and after the buck converter,

- supply behaviour during OLED initialization,

- continuity of VCC/GND wiring,

- OLED I²C communication,

- OLED module current consumption,

- and behaviour when the OLED is tested independently.

This would allow the root cause to be separated into a power problem, communication problem, wiring problem or component failure.

# 5. HW-03 — Buck Converter Output Could Not Initially Be Adjusted to Required Voltage

## 5.1 Description

During development of the battery power subsystem, the buck converter was intended to reduce the battery voltage to approximately 5 V for the downstream electronics.

However, the output voltage could not initially be adjusted to the required value.

The battery voltage was approximately 7.6 V during the relevant testing.

## 5.2 Expected Behaviour

The buck converter was expected to provide a regulated lower output voltage suitable for the downstream power architecture.

The intended target was approximately:

5 V

## 5.3 Observed Behaviour

The converter initially remained at approximately 7 V or higher despite repeated adjustment of its potentiometer.

The adjustment screw was rotated substantially—more than approximately 50 turns during the troubleshooting attempt—but the output did not reduce to the intended 5 V level.

## 5.4 Investigation

The converter's adjustment potentiometer was investigated as the primary means of controlling the output voltage.

The inability to reach the expected output suggested that further investigation of the module itself was necessary rather than simply continuing to rotate the adjustment screw indefinitely.

## 5.5 Root Cause

Not conclusively determined from the recorded testing.

Possible explanations that would require verification include:

- incorrect identification of the adjustment potentiometer,

- a damaged or unsuitable buck-converter module,

- an incorrect converter operating range,

- measurement uncertainty,

- or a module-specific adjustment/configuration issue.

A definitive root cause should not be claimed without additional testing.

## 5.6 Engineering Lesson

The issue highlighted the importance of verifying the actual electrical specifications and adjustment range of a power-converter module before integrating it into the final power architecture.

The final documentation should therefore include:

- converter input-voltage range,

- converter output-voltage range,

- maximum rated output current,

- actual configured output voltage,

- and manufacturer/module identification.

# 6. HW-04 — Power Architecture Required Multiple Development Iterations

## 6.1 Description

The HAQI-SMART power subsystem underwent several iterations during development.

The prototype needed to support:

- external DC power during development/testing,

- battery-powered operation,

- solar charging,

- voltage regulation,

- and controlled switching/power-path behaviour.

As a result, the power architecture required significant development and troubleshooting before arriving at the final arrangement.

## 6.2 Development Challenges

Several power-related questions and problems had to be addressed during development, including:

- how the solar panel should connect to the charging subsystem,

- how the battery should connect to the charging and load system,

- how the battery voltage should be regulated,

- how multiple power sources should be managed,

- how to avoid directly connecting incompatible power sources together,

- and how to provide a suitable regulated supply to the ESP32 and its peripherals.

## 6.3 Final Battery-Side Arrangement

The final battery-to-electronics arrangement confirmed by the hardware team is:

18650 Battery

↓

Buck Converter

↓

YX850 Power-Path / Switching Module

↓

ESP32 / System Electronics

The final prototype also includes a:

10 W Solar Panel

and associated solar charging subsystem.

## 6.4 Engineering Significance

The development process demonstrated that the power subsystem could not be treated as a simple battery-to-ESP32 connection.

The system required separate consideration of:

- energy generation,

- battery charging,

- battery protection,

- voltage regulation,

- power switching,

- and final load supply.

The final architecture therefore represents the result of iterative hardware development rather than the initial configuration.

# 7. General Hardware Testing Observations

During hardware development, the system was tested under different power and operating conditions.

An important lesson from the testing was that successful operation under one power source does not necessarily guarantee identical behaviour under another power source.

This was particularly evident from the OLED behaviour observed during battery-powered and DC-adapter-powered operation.

For this reason, final hardware validation should test the complete system under every intended operating condition, including:

- DC-adapter operation,

- battery operation,

- solar-assisted/battery operation,

- startup,

- continuous operation,

- and transitions between available power sources.

# 8. Corrective and Preventive Measures

For future hardware revisions, the following practices are recommended.

### 8.1 Verify Power Rails

Measure the actual voltage at the load rather than relying only on the nominal output specification of the converter.

### 8.2 Test Under Load

A converter that produces the expected voltage without a load may behave differently when the ESP32, OLED and sensors are operating simultaneously.

### 8.3 Validate Power-Source Transitions

The system should be tested when changing between:

- external DC supply,

- battery supply,

- and solar/battery operation.

### 8.4 Document Actual Module Specifications

Generic modules should be identified using:

- manufacturer,

- model,

- input range,

- output range,

- current rating,

- and other relevant specifications.

### 8.5 Check Display Supply Separately

If the OLED again shows abnormal behaviour, its supply voltage should be measured directly at the OLED module while the display is operating.

# 9. Issue Status Summary

| ID | Issue | Observed Condition | Root Cause | Status |
| --- | --- | --- | --- | --- |
| HW-01 | OLED green/distorted display | Battery operation | Not conclusively determined | Investigated |
| HW-02 | OLED later stopped operating | DC-adapter operation | Not conclusively determined | Investigated |
| HW-03 | Buck output could not initially reach 5 V | Battery/buck testing | Not conclusively determined | Investigated |
| HW-04 | Power architecture required iterations | System integration | Design-development issue rather than component failure | Addressed through architecture development |

# 10. Important Documentation Principle

The issues recorded in this document should not be interpreted as evidence that the final prototype is non-functional.

They represent problems encountered during the development and integration process.

The purpose of an engineering errata document is to preserve these observations so that:

- future team members understand what happened,

- troubleshooting does not have to start from zero,

- design decisions can be understood in context,

- and future hardware revisions can avoid repeating the same problems.

Where a root cause was not experimentally confirmed, it is intentionally recorded as “not conclusively determined.”

# 11. Final Status

The HAQI-SMART hardware development involved iterative testing of the sensing, display and power subsystems.

The most significant documented hardware issues were associated with:

- OLED behaviour under different power conditions,

- buck-converter voltage regulation,

- and the development of the final power architecture.

These issues contributed to the refinement of the hardware design and provide useful information for future revisions and troubleshooting.

Document Status: Hardware Development Record
 Revision: V1.0
 Prepared for: HAQI-SMART Hardware Documentation
 Final verification: Required before submission